Shayan's Biofeedback Math Pipeline

Provenance: Steps 0, 5, 6, 8 are supported by physiological / signal-processing literature (see Reference column and References section). Steps 1, 3, 7, 9 are statistical or signal-processing conventions. Steps 10 and 11 are engineering design choices specific to this exposure-therapy paradigm; the threshold multipliers, weights, rate parameters and dashboard mapping will be validated through pilot recordings.

Inputs: HR (BPM), HRV = RMSSD (ms), EDA (μS) — three signals streamed via LSL.

| # | Operation | Reference |
| --- | --- | --- |
| 0 | Acquisition Why: Get the participant's autonomic-nervous-system signals into the server in real time. In: Raw biosignals from the PLUX hardware via Lab Streaming Layer. Out: Three time-series streams sampled at the hardware rate (treated as 50 Hz). Three signals streamed in: HR (BPM), HRV = RMSSD (ms), EDA (μS). EDA electrodes on distal phalanges of the index and middle fingers, nondominant hand. | Boucsein et al., 2012 (placement) [3] Task Force, 1996 (RMSSD) [8] Kothe et al., 2025 (LSL) [5] |
| 1 | EMA smoothing (per signal) Why: Suppress sample-to-sample noise (sensor jitter, quantisation) without distorting the slower physiological dynamics. Each α is tuned to its signal's bandwidth — smaller α = stronger smoothing. In: Raw HR, HRV, EDA samples from Step 0. Out: Smoothed HR, HRV, EDA samples — same rate, less noise. y_t = α · x_t + (1 − α) · y_(t−1) α_HR = 0.10, α_HRV = 0.05, α_EDA = 0.05 | Convention in signal processing |
| 2 | Baseline buffering (0–120 s) Why: Capture the participant's resting physiology so everything afterwards can be measured against their personal baseline (not population norms). The 120 s window is a compromise between data sufficiency and participant comfort. In: Smoothed samples from Step 1. Out: Three per-signal buffers, ~6000 samples each at 50 Hz. Accumulate smoothed samples per signal: X = {x_1, …, x_N} | — |
| 3 | 3σ outlier removal (at t = 120 s) Why: Reject artifacts in the baseline (motion, electrode pop, sensor glitch) so the personal baseline isn't biased by transient errors. 3σ ≈ 99.7% inclusion under a normal distribution. In: Each per-signal baseline buffer from Step 2. Out: Cleaned per-signal buffers — same content, artifacts removed. X_clean = { x ∈ X : ¦x − μ_X¦ ≤ 3·σ_X } | Statistical convention |
| 4 | Compute personal baselines Why: Single per-signal reference values for this participant's resting state. From this point on, the live phase is measured relative to these. In: Cleaned baseline buffers from Step 3. Out: Three scalars: avgHR, avgHRV, avgEDA. Stored for the rest of the session. avgHR = mean(HR_clean) avgHRV = mean(HRV_clean) avgEDA = mean(EDA_clean) | — |
| 5 | Percentage deviation from baseline (HRV inverted) Why: Convert each signal into a single comparable scale (% above resting). HRV is inverted because RMSSD DROPS during stress (parasympathetic withdrawal) — inversion makes all three Δ signed positive when stressed. In: Smoothed live samples + personal baselines from Step 4. Out: Three percentage-deviation streams ΔHR, ΔHRV, ΔEDA. ΔHR = (HR − avgHR ) / avgHR · 100 ΔHRV = (avgHRV − HRV) / avgHRV · 100 (inverted) ΔEDA = (EDA − avgEDA) / avgEDA · 100 | Task Force, 1996 (RMSSD ↔ HF ↔ vagal) [8] Billman, 2013 (LF/HF critique) [2] Quigley et al., 2024 (modern HRV guidelines) [7] |
| 6 | Weighted fusion — instantaneous stress Why: Combine three physiological signals into one number. Weights reflect physiological specificity: EDA is purely sympathetic (highest weight), HRV is purely parasympathetic (mid), HR is mixed (lowest). In: ΔHR, ΔHRV, ΔEDA from Step 5. Out: Single instantaneous stress value S_instant (a number, signed: positive = stressed). S_instant = 0.5 · ΔEDA + 0.3 · ΔHRV + 0.2 · ΔHR | Posada-Quintero & Chon, 2020 (EDA solely sympathetic) [6] Baghestani et al., 2024 (recent SNS-EDA confirmation) [1] Gamboa et al., 2025 (PLUX, current literature) [4] |
| 7 | Rolling-mean smoothing (50 samples = 1 s @ 50 Hz) Why: Single-sample S_instant is jittery. Averaging over 1 second of samples removes residual noise without lagging the signal noticeably. From here on, S_t is the canonical stress measurement. In: Stream of S_instant from Step 6. Out: Smoothed stress index S_t (one value per output tick). S_t = (1/50) · Σ_{k = t−49 … t} S_instant,k | Statistical convention |
| 8 | Welford running standard deviation Why: We need σ_S to set thresholds in Step 9, but we can't store the full session history (sessions can be hours long). Welford's algorithm computes the same σ as a one-pass batch computation, but uses constant memory. In: Each new S_t from Step 7. Out: Running mean μ_S and running standard deviation σ_S, updated incrementally. n ← n + 1 δ ← S_t − μ_old μ_new ← μ_old + δ / n M₂ ← M₂ + δ · (S_t − μ_new) σ_S² ← M₂ / n | Welford, 1962, pp. 419–420 [9] |
| 9 | Threshold derivation Why: Define how big a deviation has to be before it counts as 'stressed' or 'ultra-stressed'. Multiplying σ_S by these constants yields the corresponding standard-normal percentiles, so deviations are interpretable in statistical terms. In: σ_S from Step 8. Out: Two threshold values, updated each tick. Used by Step 10. thresh_mild = 1.33 · σ_S (boundary: calm / stressed) thresh_high = 2.28 · σ_S (boundary: stressed / ultra_stressed) (≈ 90th and 99th percentile of standard normal) | Statistical convention |
| 10 | State classification + rate-based balloon control Why: This is the therapeutic core. The balloon implements graded exposure: descend when participant is overwhelmed (relief), hold when in the goal stress band, ascend when too calm (more exposure). Three difficulty modes let the operator scale the height range to the participant's tolerance. Rates scale with the range so all modes feel equally responsive. In: S_t, thresh_mild, thresh_high, mode (easy/moderate/intense from Unity). Out: State label (calm/stressed/ultra_stressed) and balloon target height y_t. Three states (all on positive side; acrophobia paradigm — going below baseline is rare): ultra_stressed if S_t > +thresh_high stressed if +thresh_mild < S_t ≤ +thresh_high (goal zone) calm if S_t ≤ +thresh_mild Difficulty mode (selected by Unity at session start): easy: y_low = 20 m, y_high = 30 m, y_mid = 25.0 m (range 10 m) moderate: y_low = 30 m, y_high = 45 m, y_mid = 37.5 m (range 15 m) intense: y_low = 45 m, y_high = 65 m, y_mid = 55.0 m (range 20 m) y_mid is the geometric midpoint and the balloon's starting height each session. Rate-based balloon update (Δt = 1/output rate): if state = ultra_stressed: y_(t+1) = y_t − k_down · Δt if state = stressed: y_(t+1) = y_t (hold) if state = calm: y_(t+1) = y_t + k_up · Δt clamped to [y_low, y_high] Rates SCALED with mode range so traversal time is constant across modes: k_down = c_down · (y_high − y_low), k_up = c_up · (y_high − y_low) c_down = 0.010 / s, c_up = 0.005 / s (asymmetric: relief faster than re-exposure) → easy: k_down=0.10, k_up=0.05; moderate: k_down=0.15, k_up=0.075; intense: k_down=0.20, k_up=0.10 (m/s) Per-frame Unity lerp toward the running y target: y_render_(t+1) = y_render_t + λ · (y_t − y_render_t), λ ∈ (0, 1] | Engineering design |
| 11 | Dashboard 0–100 score (operator display only — does NOT drive balloon) Why: Quick visual readout for the operator running the session. Maps S_t to a familiar 0–100 scale, calibrated so 50 marks the stressed boundary and 100 the ultra-stressed boundary. In: S_t and current thresholds. Out: Number 0–100 sent to Unity as the `stress` field. Cosmetic mapping for the operator's dashboard, sent to Unity as the `stress` field for backwards compatibility with existing visualisation code: if S_t ≤ thresh_mild: display = 50 · (S_t / thresh_mild) if S_t ≤ thresh_high: display = 50 + 50 · (S_t − thresh_mild) / (thresh_high − thresh_mild) if S_t > thresh_high: display = 100 (clamped) Interpretation: 0 = baseline, 50 = entering stressed zone, 100 = entering ultra-stressed zone. | Engineering design |

Outputs per tick (50 Hz): S_t,  state (ultra_stressed / stressed / calm),  y_t (balloon target),  display 0–100,  thresh_mild,  thresh_high,  smoothed HR / HRV / EDA,  baseline status,  mode (echoed from Unity).

References

[1]  Baghestani, F., Kong, Y., D'Angelo, W., & Chon, K. H. (2024). Analysis of sympathetic responses to cognitive stress and pain through skin sympathetic nerve activity and electrodermal activity. Computers in Biology and Medicine, 173, 108070.

Cited for: Step 6 — recent comparative confirmation that EDA is a well-studied non-invasive marker of sympathetic nervous system assessment, validated against direct nerve recordings under cognitive stress and pain (whole paper).

[2]  Billman, G. E. (2013). The LF/HF ratio does not accurately measure cardiac sympatho-vagal balance. Frontiers in Physiology, 4, 26.

Cited for: Step 5 (HRV justification) — argument that the LF/HF ratio cannot accurately quantify cardiac sympathovagal balance (whole paper; central thesis).

[3]  Boucsein, W., Fowles, D. C., Grimnes, S., Ben-Shakhar, G., Roth, W. T., Dawson, M. E., & Filion, D. L. (2012). Publication recommendations for electrodermal measurements. Psychophysiology, 49(8), 1017–1034.

Cited for: Step 0 — EDA electrode placement on distal phalanges of fingers, nondominant hand. §2.2 Recording Sites and Pretreatment, p. 1022.

[4]  Gamboa, P., Varandas, R., Mrotzeck, K., Plácido da Silva, H., & Quaresma, C. (2025). Electrodermal activity analysis at different body locations. Sensors, 25(6), 1762.

Cited for: Step 6 — current biosignals platform literature confirming EDA as a non-invasive measure of the sympathetic nervous system (Abstract). PLUX-affiliated paper.

[5]  Kothe, C., Shirazi, S. Y., Stenner, T., Medine, D., Boulay, C., Grivich, M. I., Artoni, F., Mullen, T., Delorme, A., & Makeig, S. (2025). The lab streaming layer for synchronized multimodal recording. Imaging Neuroscience, 3, IMAG.a.136.

Cited for: Step 0 — LSL middleware: software-based clock synchronisation and millisecond-precise timestamps (Abstract); UDP broadcast and multicast outlet discovery (§2.2.1 The discovery protocol).

[6]  Posada-Quintero, H. F., & Chon, K. H. (2020). Innovations in electrodermal activity data collection and signal processing: A systematic review. Sensors, 20(2), 479.

Cited for: Step 6 — solely sympathetic control of EDA, evidenced by simultaneous sympathetic-nerve and EDA recordings showing direct correspondence between nerve bursts and skin conductance responses (Abstract; §Basics of the Signal Analysis of EDA).

[7]  Quigley, K. S., Gianaros, P. J., Norman, G. J., Jennings, J. R., Berntson, G. G., & de Geus, E. J. C. (2024). Publication guidelines for human heart rate and heart rate variability studies in psychophysiology — Part 1: Physiological underpinnings and foundations of measurement. Psychophysiology, 61(9), e14604.

Cited for: Step 5 (HRV justification) — modern Committee Report cautioning against LF/HF as a sympathovagal balance index; cardiac sympathetic and parasympathetic activity occupy a 2-dimensional space rather than a single reciprocal axis.

[8]  Task Force of the European Society of Cardiology and the North American Society of Pacing and Electrophysiology. (1996). Heart rate variability: Standards of measurement, physiological interpretation, and clinical use. Circulation, 93(5), 1043–1065.

Cited for: Step 0 — RMSSD definition (§Measurement of HRV → Time Domain Methods → Statistical Methods). Step 5 — RMSSD inverse to sympathetic arousal, via §Components of HRV (HF reflects efferent vagal activity) combined with Table 3, Approximate Correspondence of Time Domain and Frequency Domain Methods (RMSSD ↔ HF).

[9]  Welford, B. P. (1962). Note on a method for calculating corrected sums of squares and products. Technometrics, 4(3), 419–420.

Cited for: Step 8 — incremental online algorithm for running mean and running variance without storing the full history (whole paper, pp. 419–420).